CHARMM Element doc/aspenr.doc $Revision: 1.1.1.1 $

#

File: ASPENR, Node: Top, Up: (chmdoc/commands.doc), Next: Syntax

Atomic Solvation Parameter Based Energy

Purpose: calculate solvation free energy and forces based on the

exposed surface area of each atom using Atomic Solvation Parameters.

Please report problems to brbrooks@helix.nih.gov

REFERENCES:

M. Wesson and D. Eisenberg, 19??.

* Menu:

* Syntax::      Syntax of ASP input

* Structure::   Structure of the .surf file containing ASP data

* Examples::    Usage examples of the ASP module

#

File: ASPENR, Node: Syntax, Up: Top, Previous: Top, Next: Structure

Syntax

[SYNTAX ASP functions]

Syntax: The ASP specifications can be specified any time prior to an

energy calculation and can be input either through reading a

file or parsed directly off the command line - although the

file route is more usual.  Once turned on, the ASP energy

term is in place during the course of the CHARMM run, i.e., it

cannot be turned off except using the skipe command,

see *note Skipe (chmdoc/energy.doc).

Reading surf file:  open unit 1 read vap_to_wat_kd.surf

read surf unit 1

close unit 1

#

File: ASP, Node: Structure, Up: Top, Next: Examples, Previous: Syntax

This module computes solvation energies and forces based on the

surface area model proposed by Wesson and Eisenberg, i.e.,

E_solv = Sum (Gamma_i * ASA_i + Eref_i), where Gamma_i is a parameter

describing the free energy cost of burying atom i (in units of

cal/mol/A^2), ASA_i is the surface area of atom i with radius RvdW_i

and probe radius Rprobe and Eref_i is a reference solvation energy.

The analytic expressions for atomic surface areas and corresponding

cartesian derivitives are used in these calculations.

The values of the required parameters are read from a "surf" file

which has the following syntax:

* file:  vap_to_wat_kd.surf

*

!   Note:  These are asp's from Wolfendon water to vapor numbers,

!                     adjusted for standard state by Kyte and Doolittle.

!                    They are in units of cal/(mol*A**2).

!                    Table of ASP's

!

1.400000 ! the probe radius

!

! residue-type  atom-name  asp-value  radius  reference-area  swap-pairs

ANY           H*        00.0     -1.0       0.0  ! ignore hydrogens

ANY           C         04.0      1.90      0.00

ANY          OT1      -112.0      1.40      0.00           OT2

ANY          OT2      -112.0      1.40      0.00           OT1

ANY           N       -112.0      1.70      0.00

.

.

.

.

TRP          CZ2        04.0     1.90      0.00

TRP          CZ3        04.0     1.90      0.00

TRP          CH2        04.0     1.90      0.00

ASN          OD1       -112.0    1.40      0.00

ASP          OD1       -112.0    1.40      0.00           OD2

ASP          OD2       -166.0    1.40      0.00           OD1

END

Notes: -ANY refers to any residue type

-A negative radius causes the atom to be ignored

(such as hydrogens,...)

-Atom name can use CHARMM wildcard rules (not residue names).

-These commands ar eprocessed sequentially.  If an

atom is matched by more then one line the LAST line is used.

-This file is free field format.

#

File: ACE, Node: Examples, Up: Top, Previous: Structure, Next: Top

Examples

To set up energy calculations/simulations/minimizations with the ASP potential,

the following call is expected to be adequate in most situations:

open unit 1 read form name vap_to_wat_kd.surf

read surf unit 1

close unit 1

When you do an energy calculation, dynamics or minimization with ASP, you get

columns in the log file printout with energy terms for ASP, e.g.,

ENER ENR:  Eval#     ENERgy      Delta-E         GRMS

ENER INTERN:          BONDs       ANGLes       UREY-b    DIHEdrals    IMPRopers

ENER EXTERN:        VDWaals         ELEC       HBONds          ASP         USER

----------       ---------    ---------    ---------    ---------    ---------

ENER>        0    -44.02560      7.74091      6.01738

ENER INTERN>        0.00000      0.04160      0.00000      0.00000      0.04556

ENER EXTERN>        5.95140    -42.32325      0.00000     -7.74091      0.00000

----------       ---------    ---------    ---------    ---------    ---------

and the same during minimization and dynamics.

see also: test cases c27test/aspenr.inp